Heat press setting:

- Preheat the heat press to 305°F (150°C).

- Place your fabric onto the heat press and preheat it for 5-10 seconds to remove any moisture and wrinkles.

- Place your design onto the fabric, with the carrier sheet facing up.

- Apply heat and pressure for 8-10 seconds using medium to high pressure.

- Peel off the carrier sheet while it is Cold.

- If the vinyl is not fully adhered, repeat steps 4-5 until it is.

Iron-On setting:

- For using a household iron, set it to the Cotton/Linen or Wool setting, which is usually the highest temperature setting available.

- Turn OFF the steam setting on the iron.

- Preheat the area of the fabric where you want to apply the vinyl for 10-15 seconds.

- Place the weeded image onto the preheated fabric, liner side up. Apply medium to high pressure with the iron for 25-30 seconds, pressing down firmly and evenly with both hands.

- Flip the fabric over and apply medium pressure with the iron to the back of the material for an additional 25-30 seconds.

- Let the vinyl cool for a few seconds, then gently peel off the carrier sheet while it is still Cold.

- If the vinyl is not fully adhered, repeat steps 4-6 until it is.

Care Instructions:

- Wait for 24 hours before washing the garment after applying HTV vinyl.

- Turn the garment inside out and wash it in cold water using a gentle cycle.

- Do not use bleach or any harsh cleaning agents that may damage the vinyl.

- Do not tumble dry the garment. Hang or lay it flat to air dry.

- Do not dry clean.

- If necessary, iron the garment inside out using a low or medium heat setting, avoiding direct contact with the vinyl.